| Course Name: | Elements of Electrical and Electronics Engineering | Semester: | I/II |
| --- | --- | --- | --- |
| Date of Performance: | 4/1/2022 | Batch No: | A2 |
| Faculty Name: | Maruti Zalte | Roll No: | 16010121045 |
| Faculty Sign & Date: |  | Grade/Marks: | / 25 |

Experiment No: 9

Title: Measurement of Power using Two Wattmeter Method

Aim and Objective of the Experiment:

- To measure the power of three phase power using Two Wattmeter Method

COs to be achieved:

CO1: Analyze resistive networks excited by DC sources using various network theorems.

Circuit Diagram/ Block Diagram:

Circuit Diagram

Stepwise-Procedure:

- Connect the circuit as shown in circuit diagram - Increase the load and note down the reading VL,IL,W1 and W2 - Practically you will obtain total power W=W1+W2 - Theoretically power is measured by using formula P=√3VLILcosϕ, using cosϕ=1(unity) for resistive load.

Observation Table: Sr.no; VL; IL; W1; W2; W=W1+W2; P=√3VLILCOSϕ; Load. 1; 266; 1.53; 352; 354; 706; 702.93; Resistive. 2; 379; 0.66; -53.3; 184; 130.7; 134.31; Inductive(cos ϕ=0.31). 3; 328; 1.73; -120; 414; 294; 304.67; Inductive(cos ϕ=0.31). 4; 160; 2.77; 389; 373; 763; 759.97; Reactance(cos ϕ=0.99).

Screenshot of Output: Pargat Singh Pargat Singh Pargat Singh Pargat Singh Calculations

Conclusion:

The measurement of three phase power for both, star and delta connection was successfully measured using two watt meters.

Signature of faculty in-charge with Date: